Answer for Review Questions

Ans.1

- Sequential access

- In case of sequential access head search or reach at the memory address from start address of our storage device.

- If you want to access the nth byte of a file, you must read all bytes 0 through n this is not work in random access.

- Start at the beginning and read through in order

- Access time depends on location of data and previous location

- Direct Access:-

- Access data store on our storage media or RAM directly from that memory address.

- Direct access means going straight to the record you want

- Individual blocks have unique address

- Access is by jumping to vicinity plus sequential search

- Access time depends on location of data and previous location

- Random access

- Random access means pick data randomly and then find that data which you required.

- Individual addresses identify location exactly

- Access time is independent of data location and previous location
Advantages: 
(1) it provides rapid access to the desired information. In a decision-making environment where information is needed quickly, random access is a requisite to rapid retrieval; 
(2) it is efficient for retrieving a relatively few records at a time; and 
(3) it provides a method of keeping files up to date as transactions or events occur

Ans.2

There is a trade-off among the three key characteristics of memory: cost, capacity,

and access time. That means:-

- Faster access time – greater cost per bit

- Greater capacity – smaller cost per bit

- Greater capacity – slower access time

- As one goes down the memory hierarchy: (a) decreasing cost per bit; (b) increasing capacity; (c) increasing access time; (d) decreasing frequency of access of the memory by the processor.

Ans.3

The locality principle is the phenomenon that the collection of the data locations referenced in a short period of time in a running computer.

To define this principle we can use a lot of terminology like Locality of Reference principle and other. But, this two general principle define it better:-

/////////////////////////////////////

Ans.4

Direct mapping:-

- Each block of main memory maps to only one cache line

- Direct mapping cache treats a main memory address as 3 distinct fields

These are Tag identifier, Line number identifier, Word identifier.

- No two blocks in the same line have the same Tag field

- Pros of  Direct Mapping

• Simple

- Cons of Direct Mapping

• Inexpensive

• Fixed location for a given block

If a program accesses two blocks that map to the same line repeatedly, then cache misses are very high

Associative mapping:-

- A main memory block can be loaded into any line of the cache

- A memory address is interpreted as a tag and a word field

- The tag field uniquely identifies a block of main memory

- Each cache line’s tag is examined simultaneously to determine if a block is in cache

- Main memory addresses are viewed as two fields

These are tags(s) and words (w).

- Pros of Associative Mapping

• Flexibility as to which block to replace when a new block is read into cache

- Replacement algorithms designed to maximize cache hit ratio

- Cons of Associative Mapping

• Complex circuitry required to examine the tags of all cache lines in parallel

Set Associative mapping:-

Compromise between direct and associative mapping

• Cache divided into v sets

• Each set contains k lines

• A given block maps into any line in a given set

Ans.5

Direct mapping cache treats a main memory address as 3 distinct fields

These are Tag identifier(s-r, 8bits), Line number identifier(r, 14bits), and Word (w, 2bits) identifier.

- Word identifier specifies the specific word (or addressable unit) in a cache line that is to be read

- Line identifier specifies the physical line in cache that will hold the referenced address

- The tag is stored in the cache along with the data words of the line

Ans.6

A memory address is interpreted as a tag(s, 22 bits) and a word (w, 2 bits) field. Line numbers (ids) have no meaning in the cache.

- The tag field uniquely identifies a block of main memory. Every line's tag must be examined for a match

- Least significant w bits = word position within block. The "Word" field again selects one from among 16 addressable words (bytes) within the line.)

Ans.7

- Compromise between direct and associative mapping. Divide cache into a number of sets (v), each set holding a number of lines (k).Therefore, a memory address is interpreted as a tag(s-d, 9 bits), set (d, 13 bits) and a word (w, 2 bits) field.

- Most significant s bits = tag used to identify which block is stored in a particular line of cache.

- Word service as offset for words to be accessed or word position in block.

Ans.8

- Temporal Locality: Concept that a resource will be referenced at one point in time will be referenced again. Cache miss traffic decreases fast when cache size increases and temporal locality determines sensitivity to cache size.

- Spatial Locality: Concept that likelihood of referencing a resource is higher if a resource near it was referenced. Cache miss traffic does not increase much when line size increases. Spatial locality determines sensivity to line size. And others like equidistance, branch.

Ans.9

- This can be can be capitalized by hierarchical storage hardware.

Modern cache designs exploit spatial locality by fetching large blocks of data called cache lines on a cache miss. Subsequent references to words within the same cache line result in cache hits.