## POWER6

## Introduction

The POWER6 is a microprocessor developed by IBM that implemented the Power ISA v.2.03. When it became available in systems in 2007, it succeeded the POWER5+ as IBM's flagship Power microprocessor. It is part of the eCLipz project, said to have a goal of converging IBM's server hardware where practical (hence "ipz" in the acronym: iSeries, pSeries, and zSeries).

POWER6 was described at the International Solid-State Circuits Conference (ISSCC) in February 2006, and additional details were added at the Microprocessor Forum in October 2006 and at the next ISSCC in February 2007. It was formally announced on May 21, 2007.  It was released on June 8, 2007 at speeds of 3.5, 4.2 and 4.7 GHz, but the company has noted prototypes have reached 6 GHz. POWER6 reached first silicon in the middle of 2005, and was bumped to 5.0 GHz in May 2008 with the introduction P595.

## Description

The POWER6 is a dual-core processor. Each core is capable of two-way simultaneous multithreading (SMT). The POWER6 has approximately 790 million transistors and is 341 mm2 large fabricated on a 65 nm process. A notable difference from POWER5 is that the POWER6 executes instructions in-order instead of out-of-order. This change often requires software to be recompiled for optimal performance, but the POWER6 still achieves significant performance improvements over the POWER5+ even with unmodified software, according to the lead engineer on the POWER6 project.

POWER6 also takes advantage of ViVA-2, Virtual Vector Architecture, which enables the combination of several POWER6 nodes to act as a single vector processor.

Each core has two integer units, two binary floating-point units, an AltiVec unit, and a novel decimal floating-point unit. The binary floating-point unit incorporates “many microarchitectures, logic, circuit, latch and integration techniques to achieve [a] 6-cycle, 13-FO4 pipeline,” according to a company paper. Unlike the servers from IBM's competitors, the POWER6 has hardware support for IEEE 754 decimal arithmetic and includes the first decimal floating-point unit integrated in silicon. More than 50 new floating point instructions handle the decimal math and conversions between binary and decimal. This feature was also added to the z10 microprocessor featured in the System z10.

Each core has a 64 KB, four-way set-associative instruction cache and a 64 KB data cache of an eight-way set-associative design with a two-stage pipeline supporting two independent 32-bit reads or one 64-bit write per cycle. Each core has semi-private 4 MiB unified L2 cache, where the cache is assigned a specific core, but the other has a fast access to it. The two cores share a 32 MiB L3 cache which is off die, using an 80 GB/s bus.

POWER6 can connect to up to 31 other processors using two inter node links (50 GB/s), and supports up to 10 logical partitions per core (up to a limit of 254 per system). There is an interface to a service processor that monitors and adjusts performance and power according to set parameters.

IBM also makes use of a 5 GHz duty-cycle correction clock distribution network for the processor. In the network, the company implements a copper distribution wire that is 3 µm wide and 1.2 µm thick. The POWER6 design uses dual power supplies, a logic supply in the 0.8-to-1.2 Volt range and an SRAM power supply at about 150-mV higher.

The thermal characteristics of POWER6 are similar to that of the POWER5. Dr Frank Soltis, an IBM chief scientist, said IBM had solved power leakage problems associated with high frequency by using a combination of 90 nm and 65 nm parts in the POWER6 design.

IBM introduced POWER6* microprocessor-based systems in 2007. Based upon the proven simultaneous multithreaded (SMT) implementation and dual-core technology in the POWER5* chip, the design of the POWER6 microprocessor extends IBM leadership by introducing a high-frequency core design coupled with a cache hierarchy and memory subsystem specifically tuned for the ultrahigh-frequency multithreaded cores. The POWER6 processor implements the 64-bit IBM Power Architecture* technology. Each POWER6 chip incorporates two ultrahigh-frequency dual threaded SMT processor cores, a private 4-MB level 2 cache (L2) for each processor, a 32-MB L3 cache controller shared by the two processors, two integrated

memory controllers, an integrated I/O controller, an integrated symmetric multiprocessor (SMP) coherence and data interconnect switch, and support logic for dynamic power management, dynamic configuration and recovery, and system monitoring. The SMP switch enables scalable connectivity for up to 32 POWER6 chips for a 64-way SMP. The ultrahigh-frequency core represents a significant change from prior designs. Driven by the latency and throughput requirements of the new core, the large, private L2 caches represent a departure from the designs of the POWER4* and POWER5 processors, which employed a smaller, shared L2 cache. The large, victim L3 cache, shared by both cores on the chip and accessed in parallel with the L2 caches, is similar in principle to the POWER5 L3 cache, despite differences in the underlying

implementation resulting from the private L2 caches. Likewise, the integrated memory and I/O controllers are similar in principle to their POWER5 counterparts. The SMP interconnect fabric and associated logical system topology represent broad changes brought on by the need to enable improved reliability, availability, and serviceability (RAS), virtualization, and dynamic

configuration capabilities. The enhanced coherence protocol facilitates robust scalability while enabling improved system packaging economics. In this paper, we focus on the micro architecture and its impact on performance, power, system organization, and cost. We begin with an overview of the key features of the POWER6 chip, followed by detailed descriptions of the ultrahigh-frequency core, the cache hierarchy, the memory and I/O subsystems, the SMP interconnect, and the advanced data pre-fetch capability. Next, we describe how the POWER6 chipset can be employed in diverse system organizations.

## High-frequency core design

The POWER6 core is a high-frequency design that is optimized for performance for the server market as well as power. It provides additional enterprise functions and RAS characteristics that approach mainframe offerings.

Figure 1 Evolution of the POWER6 chip structure. (SMT2: a dual-threaded simultaneous multithread.)

Its 13-FO41 pipeline structure yields a core whose frequency is two times that of the 23-FO4 POWER5 core. The function in each pipeline stage is tuned to minimize excessive circuitry, which causes delay and consumes excessive power. Speculation, which is costly at high

frequency, is minimized to prevent wasted power dissipation. As a result, register renaming and massive out-of-order execution as implemented in the POWER4 and POWER5 processor designs are not employed. The internal core pipeline, which begins with instruction fetching from the instruction cache (I-cache) through instruction dispatch and execution, is kept as short as possible. The instruction decode function, which consumed three pipe stages in the POWER5 processor design, is moved to the pre-decode stages before instructions are written into the I-cache. Delay stages are added to reduce the latency between dependent instructions. Execution latencies are kept as low as possible while cache capacities and associativity are increased. The POWER6 core has twice the cache capacity of its predecessor, providing one-cycle back-to-back fixed-point (FX) execution on dependent instructions, a two-cycle load for FX instructions, and a six-cycle floating-point (FP) execution pipe. The number of pipeline stages of the POWER6 processor design (from instruction fetch to an execution that produces a result) is similar to the POWER5 processor stages, yet the POWER6 core operates at twice the frequency of the POWER5 core. In place of speculative out-of-order execution that requires costly circuit renaming, the POWER6 processor design concentrates on providing data pre-fetch. Limited out-of-order execution is implemented for FP instructions. Dispatch and completion bandwidth for SMT has been improved. The POWER6 core can dispatch and complete up to seven instructions from both threads simultaneously. The bandwidth improvement, the increased cache capacity, cache associativity, and other innovations allow the POWER6 core to deliver better SMT speedup than the POWER5 processor-based system. Power management was implemented throughout the core, allowing a clock gating efficiency2 of better than 50%.

## Balanced system throughput

While the frequency trade-offs were appropriate for the core, it did not make sense to extend ultrahigh frequency to the cache hierarchy, SMP interconnect, memory subsystem, and I/O subsystem. In the POWER5 processor design, the L2 cache operates at core frequency, and the

remaining components at half that frequency. Preserving this ratio with the higher relative frequency of the POWER6 core would not improve performance but would actually impair it, since many latency penalties outside the core are more tied to wire distance than device

speeds. Because the latencies in absolute time tend to remain constant, incorporating a higher-frequency clock results in added pipeline stages. Given that some time is lost every cycle because of clocking overhead, the net effect is to increase total latency in absolute time while increasing the power dissipation due to the increase in pipeline stages. Therefore, for the POWER6 processor design, the L2 cache, SMP interconnect, and parts of the memory and

I/O subsystems operate at half the core frequency, while the L3 cache operates at one-quarter, and part of the memory controller operates at up to 3.2 GHz. With lower power and slower devices, chip power is reduced. Because of their lower speed relative to the core, these components must overcome latency and bandwidth challenges to meet the balanced system performance requirements. To achieve a balanced system design, all major subsystems must realize similar throughput improvements, not merely the cores. The cache hierarchy, SMP interconnects fabric, memory subsystem, and I/O subsystem must keep up with the demands for data generated by the more-powerful cores.

Table 1 POWER5 to POWER6 processor throughput comparison (relative to core cycles).

Therefore, for the POWER6 processor design, the internal data throughput was increased commensurately with the increase in processing power, as shown in Table 1.

Since the L2 cache was designed to operate at half the frequency of the core, the width of the load and store interfaces was doubled; instead of driving 32 bytes of data per core cycle into the core, the POWER6 processor L2 drives an aggregate of 64 bytes every other core cycle. While the POWER5 processor L2 can obtain higher peak bandwidth when simultaneously delivering data to the data cache (D-cache) and I-cache, in realistic situations the D-cache and I-cache do not drive high-throughput requirements concurrently. This is because high bus utilization due to D-cache misses typically occurs in highly tuned single-threaded scenarios when there are

multiple outstanding load instructions continuously in the pipeline, while I-cache misses interrupt the flow of instructions into the pipeline. Instead of accepting 8 bytes of store data per core cycle, the POWER6 processor L2 accepts 16 bytes of store data every other core cycle. Note that the aggregate bandwidth of the POWER6 processor L2 per core per cycle is two thirds

that of the POWER5 processor L2. It does not have to scale perfectly for the following reasons: The POWER6 core has larger L1 caches, so there are fewer L1 misses driving fetch traffic to the L2; the POWER6 processor L2 can manage store traffic with 32-byte granularity, as opposed to 64-byte granularity for the POWER5 processor L2, so normally there is less L2 bandwidth expended per store. In addition, the POWER6 processor L2 is much larger per core than the POWER5 processor L2, so there are fewer L2 misses, driving fewer cast out reads and allocate writes. (The term cast out refers to the movement of deallocated, modified data from a given level in the cache hierarchy either to the next level of cache or to memory.)

For the POWER6 chip, the IBM Elastic Interface (EI) logic, which is used to connect to off-chip L3 cache data chips, I/O bridge chips, and SMP connections to other POWER6 chips, was accelerated to operate at one-half of the core frequency, keeping pace with corresponding

interfaces in prior designs by achieving significantly higher frequency targets. The POWER6 processor L3 cache can read up to 16 bytes and simultaneously write up to 16 bytes every other core cycle, just as the POWER5 processor L3. The POWER6 processor off-chip SMP interconnect comprises five sets of links. The organization of these is described later in the section ‘‘SMP interconnect.’’ Each set can import up to 8 bytes and simultaneously export up

to 8 bytes of data or coherence information every other core cycle. While this does not match the POWER5 processor SMP interconnect bandwidth per core cycle as seen by a given chip, the difference in system topology (described later in the section ‘‘SMP interconnect’’) and an increased focus on hypervisor and operating system optimizations for scalability drive a relaxation for the demand for interconnect data bandwidth. The EI logic used for connectivity to off-chip memory buffer chips was accelerated to operate at 3.2 GHz when interacting with 800-MHz DRAM (dynamic random access memory) technology. By using both integrated memory controllers, a single POWER6 chip can read up to 16 bytes of data and simultaneously write up to 8 bytes of data or commands at 3.2 GHz. The I/O controller can read up to 4 bytes and simultaneously write up to 4 bytes of data to an I/O bridge chip every other core cycle.

## Processor core

The POWER6 core micro-architecture was developed to minimize logic content in a pipeline stage. Circuit area and speculative work are minimized in order to reduce wasted power dissipation. The result is a 13-FO4 design with short pipeline, large split L1 instruction, and D-caches supporting two-way SMT. Additional virtualization functions, decimal arithmetic, and vector multimedia arithmetic were added. Checkpoint retry and processor sparing were implemented. Instruction fetching and branch handling are performed in the instruction fetch pipe (Figure 2). Instructions from the L2 cache are decoded in pre-code stages P1 through P4 before they are written into the L1 I-cache.

Figure 3

Figure 2 POWER6 core pipeline. (AG: address generation; BHT: branch table access and predict; BR: branch; DC: data-cache access; DISP: dispatch; ECC: error-correction code; EX: execute; FMT: formatting; IB: instruction buffer; IC0/IC1: instruction-cache access; IFA: instruction fetch address; ISS: issue; P1–P4: pre-decode; PD: post-decode; RF: register file access.)

Branch prediction is performed using a 16K-entry branch history table (BHT) that contains 2 bits to indicate the direction of the branch. Up to eight instructions are then fetched from the L1 I-cache and sent through the instruction decode pipeline, which contains a 64-entry instruction buffer (I-buffer) for each thread.

Instructions from both threads are merged into a dispatch group of up to seven instructions and sent to the appropriate execution units. Branch and logical condition instructions are executed

in the branch and conditional pipeline, FX instructions are executed in the FX pipeline, load/store instructions are executed in the load pipeline, FP instructions are executed in the FP pipeline, and decimal and vector multimedia extension instructions are issued through the FP issue queue (FPQ) and are executed in the decimal and vector multimedia execution unit. Data generated by the execution units is staged through the checkpoint recovery (CR) pipeline and saved in an error-correction code (ECC)-protected buffer for recovery. The FX unit (FXU) is designed to execute dependent instructions back to back.

The FP execution pipe is six stages deep. Figure 4 shows the pipeline for both the POWER6 and the POWER5 design with cycle-time delay for each stage, starting with instruction fetching from the I-cache to the time the result is available for subsequent instruction. FX load/store instructions are executed in order with respect to each other, while FP instructions are decoupled from the rest of the other instructions and allowed to execute while overlapping with subsequent load and FX instructions. Additional emphasis was put in the design to minimize the memory effect: prefetching to multiple levels of caches, speculative execution of instructions to prefetch data into the L1 D-cache, speculative prefetching of instructions into the L1 I-cache, providing a load-data buffer for FP load instructions, hardware stride prefetching, and software-directed prefetching. Buffered stages are added between the dispatch stage and the execution stage to

optimize certain execution latency between categories of instructions. The following are examples:

- The FX instructions are staged for two additional cycles prior to execution in order to achieve a one- cycle load-to-use between a load instruction and a dependent FX instruction.

Figure 3 Internal POWER6 processor pipeline compared with the POWER5 processor pipeline with cycle time.

- Branch instructions are staged for two additional cycles to line up with FX staging instructions in order to avoid an additional branch penalty on an incorrect guess.

- FP instructions are staged through the FPQ for six cycles to achieve a zero-cycle load-to-use instruction between a load instruction and the dependent FP instruction.

The POWER6 core contains several units. The instruction fetch unit (IFU) performs instruction

fetching, instruction pre-decoding, branch prediction, and branch execution. The instruction dispatch unit (IDU) performs instruction dispatch, issuing, and interrupt handling. The two FXUs, the two binary FP units (BFUs), the decimal FP unit (DFU), and the vector multimedia extension (VMX) unit are responsible for executing the corresponding set of FX, FP, decimal, and VMX instructions. The two load/store units (LSUs) perform data fetching. The recovery unit (RU) contains the data representing the state of the processor that is protected by ECC so that the state of the processor can be restored when an error condition is detected.

## POWER6+

The slightly enhanced POWER6+ was introduced in April 2009, but had been shipping in Power 560 and 570 systems since October 2008. It added more memory keys for secure memory partition, a feature taken from IBM's mainframe processors.

## Products

As of 2008[update], the range of POWER6 systems includes "Express" models (the 520, 550 and 560) and Enterprise models (the 570 and 595). The various system models are designed to serve any sized business. For example, the 520 Express is marketed to small businesses while the Power 595 is marketed for large, multi-environment data centers. The main difference between the Express and Enterprise models is that the latter include Capacity Upgrade on Demand (CUoD) capabilities and hot-pluggable processor and memory "books". All Power systems are noted for their excellent scalability and storage capabilities.

| IBM POWER6 servers |  |  |  |
| --- | --- | --- | --- |
| Name | Number of sockets | Number of cores | CPU clock frequency |
| 520 Express | 2 | 4 | 4.2 GHz or 4.7 GHz |
| 550 Express | 4 | 8 | 4.2 GHz or 5.0 GHz |
| 560 Express | 8 | 16 | 3.6 GHz |
| 570 | 8 | 16 | 4.4 GHz or 5.0 GHz |
| 570 | 16 | 32 | 4.2 GHz |
| 575 | 16 | 32 | 4.7 GHz |
| 595 | 32 | 64 | 4.2 GHz or 5.0 GHz |

IBM also offers four POWER6 based blade servers. Specifications are shown in the table below.

| IBM POWER6 blade servers |  |  |  |
| --- | --- | --- | --- |
| Name | Number of cores | CPU clock frequency | Blade slots required |
| BladeCenter JS12 | 2 | 3.8 GHz | 1 |
| BladeCenter JS22 | 4 | 4.0 GHz | 1 |
| BladeCenter JS23 | 4 | 4.2 GHz | 1 |
| BladeCenter JS43 | 8 | 4.2 GHz | 2 |

All blades support AIX, i, and Linux. The BladeCenter S and H chassis is supported for blades running AIX, i, and Linux. The BladeCenter E, HT, and T chassis support blades running AIX and Linux but not i.

At the SuperComputing 2007 (SC07) conference in Reno a new water-cooled Power 575 was revealed. The 575 is composed of 2U "nodes" each with 32 POWER6 cores at 4.7 GHz with up to 256 GB of RAM. Up to 448 cores can be installed in a single frame.

| IBM POWER6 disk storage |  |  |  |
| --- | --- | --- | --- |
| Name | Number of cores | CPU clock frequency | Number of controllers |
| DS8700 | 2, 4 | 4.7 GHz | 1, 2 |
| DS8800 | 2, 4, 8 | 5.0 GHz | 1, 2 |

## Summary

With its high-frequency core architecture, enhanced SMT capabilities, balanced system throughput, and scalability extensions, the POWER6 microprocessor provides higher levels of performance than the predecessor POWER5 microprocessor-based systems while offering greater flexibility in system packaging trade-offs. Additionally, improvements in functionality, RAS, and power management have resulted in valuable new characteristics of POWER6 processor-based systems.